Roll No...............

DRONACHARYA COLLEGE OF ENGINEERING, GURGAON

2nd Sessional Examination April 2013

( MDU, Rohtak)

Semester- IV					         						Branch: Civil

QUESTION PAPER

Subject with Code:	DESIGN OF CONCRETE STRUCTURES 	(CE-206-F)

Time:- 3 hours                                                                           					Max Marks:- 100

Note: Q No-1 is compulsory. Attempt one question from each Section. All questions carry equal marks.

Q1 . Objective type/ short-answer type questions.(2 x 10 = 20 marks)

- What are the stability requirements for a Retaining Wall?

- Define effective height of a column.

- State the factors on which the type of foundation to be used in a given situation depend.

- Distinguish between a beam and a slab.

- Define Nominal cover.

- Differentiate between Anchorage Bond and Deflection Bond

- Define the characteristic strength of Concrete and Steel.

- What is water-cement ratio and how does it influence the strength of concrete?

- What are the different types of cement?

- What is an admixture? What are the different types of admixtures?

SECTION-A

Q2.	State the various forms in which the reinforcement steel is available. Draw the stress-strain curve for mild steel and high strength deformed bars.

Q3.	Write short notes on:

- Different methods of R.C.C. design

- Effect of varying percentages of steel on the Moment of Resistance of R.C.C. sections

SECTION-B

Q4.	(a) State the conditions under which doubly-reinforced beams are used.

(b) Design a rectangular beam section of 300 mm width and 535 mm overall depth which is subjected to ultimate moments of 150 KN-m .Consider concrete of grade M20 and steel of grade Fe415.

Q5.	(a) What are the criteria for providing minimum and maximum reinforcement in a beam?

(b) A rectangular beam section of 300 mm width and 500 mm effective depth is reinforced with 4 bars of 20 mm dia. Determine the shear reinforcement required to resist shear forces of 300 KN. Consider concrete of grade M20 and steel of grade Fe415.

SECTION-C

Q6.	(a) Define reinforcement splicing. What is its purpose?

(b) A simply-supported T-beam of 6 m span has an effective flange width of 1.25 m, thickness of flange as 15 cm, breadth of web as 20 cm and effective depth as 35 cm. It carries a factored bending moment of 300 KN-m at its mid span. Concrete is of grade M20 and steel is of grade Fe415. Check the beam for deflection.

Q7.	Design a two-way slab for a room 5.5 m x 4.0 m clear in size if the superimposed load is 5 KN/m2. Use M15 mix and Fe415 grade steel. Edges simply supported- corners not held down

SECTION-D

Q8. 	Design a short column, square in section, to carry an axial load of 2000 KN using (i) mild steel (ii) Fe415 grade steel and M20 mix.

Q9.	Design a Counterfort Retaining Wall to retain earth 4m above ground level. The top of the earth is to be level. The density of earth is 15KN/m2. The angle of internal friction of soil is 30°. The safe bearing capacity is 200 KN/m2 and coefficient of friction between soil and wall is 0.6.

************************