A PROJECT REPORT ON

AN ENGINE DRIVEN TRICYCLE FOR DISABLED

Submitted in partial fulfilment of the requirements

for the award of degree of

BACHELOR OF TECHNOLOGY

in

MECHANICAL ENGINEERING

Submitted by:

RAKHIL KRISHNAN R

REJIN CHANDRAN R

SANEESH KUMAR T S

SATYA SHANKER M

SREEJITH V

- DEPARTMENT OF MECHANICAL ENGINEERING

- COLLEGE OF ENGINEERING TRIVANDRUM

- April 2011

A PROJECT REPORT ON

- Certificate

Certified that project report entitled “AN ENGINE DRIVEN TRICYCLE FOR DISABLED” is a bonafide work carried out in the final semester by “RAKHIL KRISHNAN R (07 400  021), REJIN CHANDRAN R (07 400 024), SANEESH KUMAR T S (07 400 026), SATYA SHANKER M (07 400 027), SREEJITH V (07 400 031)” in partial fulfilment of the requirement for the award of Bachelor of Technology in “MECHANICAL ENGINEERING” from University of Kerala during academic year 2010 - 2011, who carried out the project work under the guidance and no part of this work has been submitted earlier for the award of degree.

- S. Manoharan Achari							     Dr. Baiju B

- Professor, Dept of Mech. Engg.		        Asst. Professor, Dept of Mech. Engg.

- College of Engineering, 					        College of Engineering, Thiruvananthapuram					           Thiruvananthapuram

- A. Farook									     T. C. Rajan

- Asst. Professor, Dept of Mech. Engg.	        Asst. Professor, Dept of Mech. Engg.

- College of Engineering, 					        College of Engineering, Thiruvananthapuram					           Thiruvananthapuram

Prof. E. Abdul Rasheed, Head of Department

Department of Mechanical Engineering

College of Engineering, Thiruvananthapuram

- ACKNOWLEDGEMENT

First of all we thank the almighty for providing me with the strength and courage to present the seminar.

We avail this opportunity to express our sincere gratitude to Prof. Manoharan Achari, Prof. T. C. Rajan, Dr. Baiju and Prof. Farook of Department of Mechanical Engineering for their inspiring assistance, encouragement and useful guidance and to make our project something of real value to us and our colleagues.

We are thankful to Prof. E. Abdul Rasheed, Head of Department Mechanical Engineering, Prof. Meerakumari, Head, Part-time Course for their kind co-operation.

We are also indebted to all the teaching and non-teaching staff of the Department of Mechanical Engineering for their cooperation and suggestions, which is the spirit behind this report. Last but not the least, we express our sincere gratitude from the depth of our heart to our parents, friends and all well wishers for their kind support and encouragement in the successful completion of our project report work.

- RAKHIL KRISHNAN R, REJIN CHANDRAN R, SANEESH KUMAR T S, SATYA SHANKER M, SREEJITH V

- PROJECT TEAM

Guided By

Dr. Baiju B, Asst. Professor, Mechanical Engg. Dept, CET

Team Members:

Rakhil Krishnan R 	(07400021)

Rejin Chandran R 	(07400023)

Saneesh Kumar T S 	(07400026)

Satya Shanker M 	(07400027)

Sreejith V 			(07400031)

ABSTRACT

For the growing number of people using personal mobility devices, development of devices that address their unique needs are fundamental to their quality of life. Traditionally those with mobility impairments have used wheelchairs to participate in activities. Two problems with traditional wheelchairs are the stress they put on the user’s upper limbs and their inability to actively engage the lower limbs. In this report we outline our proposal for a mobility device that requires substantially less force to drive by exploiting the mechanical advantage that an engine provides.

The goal of the Tricycle Project is to bring increased mobility to disabled persons. Presently, hand-powered tricycles are used by many of the disabled in this community, but some current users of the hand-powered tricycles do not have the physical strength or coordination to propel themselves on the tricycle with their arms and hands. The aim of this project is to add an internal combustion engine to the current hand-powered tricycle to provide tricycle users with improved levels of mobility, facilitating freedom in travel and contribution to the community. The design objectives required a simple and affordable design, a design that needed to be reliable, sustainable, and functional.

The most important part of the design is the incorporation of the fuel powered system which was added to improve the efficiency of the tricycle and to make life more comfortable for the physically challenged persons. A spark ignition automatic single speed two- stroke engine was chosen for this design and consideration was also given to the weight of the user in which a maximum weight of 70kg was used.

CONTENTS

- Introduction

- Project Schedule

- Existing Design

- Description

- Literature Review

- Realization Plan for the Project

- Preliminary Design

- Conceptual 3D Model

- Design process

  - Design of frame

  - Design of shaft

- List of figures

- Conclusion

- References

INTRODUCTION

Hand-powered tricycles are presently being used to provide mobility for disabled persons in rural communities. With this project we designed and manufactured a system to convert the hand powered tricycle to an engine version. We essentially created an affordable, rugged tricycle.

We have worked to make our design appropriate to the culture where it will be used. This meant designing for the use of locally available parts and manufacturing capabilities. The result is a system that can be almost entirely replicated, with the exception of an engine with familiar parts, tools, and processes. Using the hand-powered tricycle as the basis for our design made the tricycle more of an appropriate technology because it uses a familiar, locally available platform as a starting point. 	Hand-powered tricycles are being used like conventional wheelchairs with the motive force coming from a person pushing from behind. This engine powered tricycle which we have designed should enable to be independently mobile.

PROJECT SCHEDULE

EXISTING DESIGN

To understand the current devices available in the area of personal mobility devices, the products can be classified in three groups:

(1) Traditional Wheelchairs,

(2) Electric Wheelchairs,

(3) Hand cycles.

The first of these products, the traditional wheelchair, is driven by the rims on two large wheels at the user’s side. This propulsion mechanism can lead to upper body injuries.

The second of these products, the powered wheelchair or scooter, only makes up less than 10% of the mobility device market.

Finally, the third type of product, the hand cycle is driven by a crank with a drive chain used to power the front wheel. These devices have three wheels, positioned like a tricycle. The crank mechanism allows users to obtain faster speeds while putting less stress on their shoulders and back. Furthermore, the tricycle design enables the device to traverse tougher terrain. Hand cycles are available as attachments to traditional wheelchairs, as well as autonomous devices.

DESCRIPTION

The design of the engine powered Tricycle is adaptable to the current hand-powered tricycles with little modification. The design consists of an IC Engine, a drive system. See picture below for design schematic:

Figure 1 - Schematic of Tricycle

The first aspect of our design that was addressed was the drive system or means of power transmission. Power must be transmitted from an IC engine to a rear wheel of the tricycle. The controls for engine speed and braking were incorporated into a simple mechanical system to facilitate operation by users with limited dexterity. The hand-power system was replaced with a steering system that disables the hand-power capability of the tricycle. Third, power is supplied to the rear wheels by means of reduction gears.

All the above components were designed to be able to be installed on the existing hand-powered tricycles with slight modification. Everything necessary to convert a hand-powered tricycle to the engine tricycle is simple to install.

LITERATURE REVIEW

Research was done online as well as in magazine articles in search of presently available solutions to our problem. We found many products that were available for purchase, but they didn’t entirely meet the requirements of our unique problem. The problem has been solved, and in many different ways, but what we found, or rather didn’t find, was a solution to our problem that meets our specific needs of affordability and appropriateness. The advantage of finding these solutions is that we can see what works, what has been tried, and what’s available on the market. Then we can more effectively consider how to design a similar product that meets our unique needs.

The figure shows different ways, very expensive ways, of hand powering a tricycle. This option may or may not allow for a better design for attaching an engine to. We decided that although these are great designs, their purpose was for recreation and would not suit the needs of the people that we are designing the kit for. Their low position doesn’t allow a good seating position for a table or clearance enough for the conditions of the area.

Figure 2 - Image

REALIZATION PLAN FOR THE PROJECT

The realisation plan for the tricycle is shown below:

BASIC IDEA

FABRICATION ASSEMBLY DRAWING

FABRICATION & QUALIFICATION COMPONENTS

FEASIBILITY STUDY

PRELIMINARY LITERATURE SURVEY

CONCEPTUAL DESIGN

FINAL DESIGN

REVIEW OF DESIGN

IDENTIFICATION OF STD. COMPONENTS

FINAL DESIGN DRAWINGS

3D MODELLING

TRAIL ASSEMBLY

FINAL ASSEMBLY

TRAIL RUN

PROCUREMENT & QUALIFICATION OF STD. COMPONENTS

FINAL TRAIL RUN

PROJECT REPORT PREPARATION

DEMONSTRATION & SUBMISSION OF PROJECT WORK

Figure 3 - Plan

PRELIMINARY DESIGN

Figure 4 – Line Diagram of concept

Figure 5 – Concept Frame

CONCEPTUAL 3D MODEL

Figure 6 – 3D Model of Frame

MAJOR COMPONENTS AND DESIGN PROCESS

- Frame

- Power Generating Unit

- Power Transmission to the wheels

a. Main Shaft

b. Sprockets

c. Chain (3/8”)

d. Counter Shaft

DESIGN OF FRAME

Section selected for frame

- Hollow Square Section 25x25x2 mm thick

- Moment of inertia of section

= 	BD3/12-bd3/12

=	(25x253/12) – (21x213/12)

=	16345.33mm4

- Pipe diameter 25x2 thick

- Moment of inertia of section

=	(D4/64) – (d4/64)

=	(254/64) – (214/64)

=	9628.20mm4

- Section modulus of section (1)

=	16345.33/12.5

=	1307.62mm3

- Section modulus of section (2)

=	9628.20/12.5

=	770.26mm3

- Design of members (Hollow section)

Maximum bending moment 		= Mmax

Safe working stress in bending		= 60N/mm2

Mmax/1307.62 				= 60 N/mm2

Mmax 					= 1307.62x60

= 78457.2 Nmm

- In case of Cantilever Beam

Mmax =	WxI,	Y=	-Fl3/3EI

Mmax =	FL/4,	Y=	Fl3/48EI

Mmax =	78457.2/80x9.81 = 100mm

Y 	=	80*9.81*1003/

(3*2.1*105*16345.3)

=	0.076mm

Mmax =	78457.2 = 80*9.81/4

L 	=	78457.2*4/(80*9.81)

Y 	=	80*9.81*4003/ (48*2.1*105*16345.33)

=	0.303mm (400mm Single span)

(a) For Hollow Section:-

- Max span possible between two supports (SSB)		=	400mm

- Max span possible between support and end cantilever 	=	100mm

- Maximum deflection in case of cantilever 	   		=	0.076mm (0.167m possible)

- Max deflection in case of simply supported beam 	   	=	0.303mm (max possible 0.67mm)

(b) Pipe section

Mmax / 770.26 		=	60N/mm2

Mmax 			=	46215.60mm

Y 				=	WL3/3EI

=	80*9.81*593 /  (3*2.1*105*9628.20)

L				=	46215.80/80*9.891

=	59mm (Cantilever)

Mmax				=	46215.6

L				=	46215.6*4/(80*9.81)

=	235.5mm

For pipe section:-

- Max span possible between two supports (SSB)	=	235.5mm

- Max span possible between end support and free end (Cantilever) = 59mm (0.098mm)

- Max deflection (Cantilever) = 2.66e-3 mm

- Max deflection (SSB) 	= 0.106mm (0.392mm)

DESIGN OF SHAFT

A shaft is a rotating member usually of circular cross section (solid or hollow), which is used to transmit power and rotational motion.

Elements such as gears, pulleys (sheaves), flywheels, clutches, and sprockets are mounted on the shaft and are used to transmit power from the driving device (motor or engine) through a machine.

The rotational force (torque) is transmitted to these elements on the shaft by press fit, keys.

The shaft rotates on ball bearings.

Various types of retaining rings, thrust bearings, grooves and steps in the shaft are used to take up axial loads and locate the rotating elements

The design of the shaft is based on the strength. The following loads have been considered for shaft design:

- Torsion due to transmitted torque

- Bending from transverse loads (gears, sprockets, pulleys/sheaves)

Shaft Material - Mild Steel

Assumptions

Weight of the vehicle 	=	50Kg

Weight of the passenger 	=	70Kg

Total weight of the vehicle 	=	120Kg

Slope of Kerala Roads 	=	1:6 (~0.2)

Figure 7 – Shaft

Design Process

The action of forces on the shaft is Torsion. The torsion formula for shafts of circular section is given by

T/J = fs/R = Cθ/L

where,	T = torque transmitted by the shaft, Nm

J = Polar Moment of inertia, mm4

fs = shear stress induced, N/mm^2

R = radius of shaft, mm

Θ = angle of twist, radians

C = modulus of rigidity, N/mm^2

L = Length of shaft, mm

Slope, tan Θ	= 	1/6S

Θ 	=	tan-1 (1/6)

=	9.46 deg

=	~ 10deg

Required Pull	= 	sin Θ x mass x g

= 	sin 10 x 120 x 9.81

=	204.42 N

Wheel Diameter 	=	0.7m

Torque required 	= 	pull x radius of wheel

= 	204.52 x  0.35

= 	71.55 Nm

Torque, T 		 = 	π/16 x fs x d3

where,

fs, shear stress =	210 MPa

factor of safety  =	2

d3 			 =	71.55 x 16 x 2 / (π x 					210e6)

d 			 = 	0.019 m

= 	19mm

Dia of shaft	~= 	20mm

Deflection of Shaft

Deflection, δ 	= 	584TL/Gd4

where,

T, Torque		=	72Nm

Length		=	160mm

δ 			=	584 x 72e3 x 160 / 100e3 x 204

δ 			= 	0.42mm

=	ie, < 1/600 l

ENGINEERING DRAWING FOR FABRICATION

Frame

Engine Shaft

Plummer Block

LIST OF FIGURES

- Figure 1 - Schematic of Tricycle

- Figure 2 - Image

- Figure 3 - Plan

- Figure 4 – Line Diagram of concept

- Figure 5 – Concept Frame

- Figure 6 – 3D Model of Frame

- Figure 7 – Shaft

CONCLUSION

Hand-powered tricycles are presently being used to provide mobility for disabled persons in rural communities. With this project we designed and manufactured a system to convert the hand powered tricycle to an engine version. We essentially created an affordable, rugged tricycle.

We have worked to make our design appropriate to the culture where it will be used. This meant designing for the use of locally available parts and manufacturing capabilities. The result is a system that can be almost entirely replicated, with the exception of an engine with familiar parts, tools, and processes. Using the hand-powered tricycle as the basis for our design made the tricycle more of an appropriate technology because it uses a familiar, locally available platform as a starting point. 	Hand-powered tricycles are being used like conventional wheelchairs with the motive force coming from a person pushing from behind. This engine powered tricycle which we have designed should enable to be independently mobile.

REFERENCES

- Assessment of Wheelchair Technology in Tanzania, Amos G. Winter, V, PhD Student, Department of Mechanical Engineering, Massachusetts Institute of Technology, International Journal for Service Learning in Engineering, Vol. 2, No. 1, pp. 60-77, Fall 2006, ISSN 1555-9033

- ADA Standards For The Accessible Design Pt. 36.

- Burke, R.J. (1999), Disability and women’s work experiences: An exploratory study, International Journal of Sociology and Social Policy, 19(12), 21-33.

- Case et.al. (2001), Virtual fitting trials in ‘design for all’, Journal of Materials Processing Technology, 117, 255-261.

- Kochan, A. (1996), Remploy: disabled and thriving, Assembly Automation, 16(1), pp. 40-41.

- Krovt et.al. (1994), Design of a walking wheelchair for the motor disabled, Proceedings of the 4th InternationalConference on Rehabilitation Robotics, Wilmington D.E., 125-130.

- Nowak, E. (1996), The role of anthropometry in design of work and life environments of the disabled population, International Journal of Industrial Ergonomics, 17, 113-121.

- Sanders, D. and Stott, I. (1999), A new prototype intelligent mobility system to assist powered wheelchair users, Industrial Robot: An International Journal, 26(6), 466-475.

- Power-Assisted Tricycle, Designers: Michael Blumenshine, Andrea Beck, Ryan Smoot, Joseph Kalinowski, Brinton Hinz, Client Coordinator: Arlene Gregory, Supervising Professor: Dr. Jerry Cupal, Dr. David Walrath, Department of Electrical and Computer Engineering, University of Wyoming, Laramie, WY 82070