ABSTRACT

In this report, a new six-stroke diesel engine is proposed and the thermodynamic performances of this engine are numerically and experimentally analyzed. Since the six-stroke diesel engine introduced here has two combustion processes in one cycle, it offers new methods of combustion control which can't be attained in an ordinary four-stroke diesel engine. In the analysis, we use a simple single zone thermodynamic model with considering the Wiebe's function for heat release rate and the Foschini’s equation for heat transfer coefficient. As a results, it was confirmed when the heat release of 1st combustion stroke and 2nd combustion stroke were equivalent, the maximum in-cylinder gas temperature was the lowest, and further it was lower than that of the four stroke diesel engine.

INTRODUCTION

Diesel engines have wide range of utilization for automobiles, locomotives & marines and co-generation systems. However, large problem is still related to undesirable emission.

To improve exhaust emissions from DI diesel engines, a new concept of Six Stroke Engine has been proposed. This engine has a second compression and combustion processes before exhaust process.

As the fuel in one cycle was divided into two combustion processes and the EGR effect appeared in the second combustion process, the decreased maximum cylinder temperature reduced NO concentration in the exhaust gas It was further confirmed that soot formed in the first combustion process was oxidized in the second combustion process .Therefore, a six stroke DI diesel engine has significant possibilities to improve combustion process because of its more controllable factors relative to a conventional four-stroke engine.

Since the cylinder temperature before the second combustion process is high because of an increased temperature in the first combustion process, ignition delay in the second combustion process should be shortened. In addition, typically less desirable low cetane number fuels might also be suitable for use in the second combustion process, because the long ignition delays of these fuels might be improved by increased cylinder temperatures from the first combustion process.

Methanol was chosen as the fuel of the second combustion. The cetane number of methanol is low and it shows low ignitability. However, since methanol will form an oxidizing radical (OH) during combustion, it has the potential to reduce the soot produced in the first combustion process.

WORKING OF SIX STROKE DIESEL ENGINE

The cycle of this engine consists of six strokes:

- Intake stroke

- First compression stroke

- First combustion stroke

- Second compression stroke

- Second combustion stroke

- Exhaust stroke

INTAKE / SUCTION STROKE:

To start with the piston is at or very near to the T.D.C., the inlet valve is open and the exhaust valve is closed. A rotation is given to the crank by the energy from a flywheel or by a starter motor when the engine is just being started. As the piston moves from top to bottom dead centre the rarefaction is formed inside the cylinder i.e. the pressure in the cylinder is reduced to a value below atmospheric pressure. The pressure difference causes the fresh air to rush in and fill the space vacated by the piston. The admission of air continues until the inlet valve closes at B.D.C.

FIRST COMPRESSION STROKE:

Both the valves are closed and the piston moves from bottom to top dead centre. The air is compressed up to compression ratio that depends upon type of engine. For diesel engines the compression ratio is 12-18 and pressure and temperature towards the end of compression are 35-40 kgf/cm2 and 600-700 0C

FIRST COMBUSTION STROKE:

This stroke includes combustion of first fuel (most probably diesel) and expansion of product of combustion. The combustion of the charge commences when the piston approaches T.D.C.

Here the fuel in the form of fine spray is injected in the combustion space. The atomization of the fuel is accomplished by air supplied. The air entering the cylinder with fuel is so regulated that the pressure theoretically remains constant during burning process.

In airless injection process, the fuel in finely atomized form is injected in combustion chamber. When fuel vapors raised to self ignition temperature, the combustion of accumulated oil commences and there is sudden rise in pressure at approximately constant volume. The combustion of fresh fuel injected into the cylinder continues and this ignition is due to high temperature developed in engine cylinder.   However this latter combustion occurs at approximately constant pressure.

Due to expansion of gases piston moves downwards. The reciprocating motion of piston is converted into rotary motion of crankshaft by connecting rod and crank. During expansion the pressure drop is due to increase in volume of gases and absorption of heat by cylinder walls.

SECOND COMPRESSION STROKE:

Both the valves are closed and the piston moves from bottom to top dead centre. The combustion products from the first compression stroke are recompressed and utilized in the second combustion process before the exhaust stroke. In typical diesel engine combustion the combustion products still contains some oxygen.

SECOND COMBUSTION STROKE:

This stroke includes combustion of second fuel having low cetane (Cetane number of fuel is defined as percent volume of cetane (c16h34) in a mixture of cetane and alpha-methyl-naphthalene that produces the same delay period or ignition lag as the fuel being tested under same operating conditions on same engine). The combustion of the charge commences when the piston approaches to T.D.C.

The second fuel injected into recompressed burnt gas can be burnt in the second combustion process. In other words combustion process of  the second fuel took place in an internal full EGR (Exhaust Gas Recirculation) of the first combustion.

This second combustion process was the special feature of the proposed Six Stroke DI Diesel Engine.

EXHAUST STROKE:

The exhaust valve begins to open when the power stroke is about to complete. A pressure of 4-5 kgf/cm2 at this instant forces about 60% of burnt gases into the exhaust manifold at high speed. Much of the noise associated with automobile engine is due to high exhaust velocity. Some of the burnt gases are however left in the clearance space. The exhaust valve closes shortly after T.D.C.

The inlet valve opens slightly before the end of exhaust stroke and cylinder is ready to receive the fresh air for new cycle.

Since from the beginning of the intake stroke the piston has made six strokes through the cylinder (Three up And Three down). In the same period crank shaft has made three revolutions. Thus for six stroke cycle engine there are two power strokes for every three revolutions of crank shaft.

PERFORMANCE ANALYSIS

MODIFICATION OVER FOUR STROKE DIESEL ENGINE:

Schematic constructions of a proposed six-stroke diesel engine and a conventional four-stroke diesel engine are shown in Fig. 2. This six-stroke diesel engine was made from a conventional four-stroke diesel engine with some modification. A sub-shaft was added to the engine, in order to drive a camshaft and injection pumps. The rotation speed of the sub-shaft was reduced to 1/3 of the rotation of an output shaft. To obtain similar valve timings between a four-stroke and a six-stroke diesel engine, the cam profile of the six-stroke diesel engine was modified. In order to separate the fuels, to control each of the injection timings and to control each injection flow rate in the first and the second combustion processes, the six-stroke diesel engine was equipped with two injection pumps and two injection nozzles. The injection pumps were of the same type as is used in the four-stroke diesel engine.

Details of nozzle position of the four-stroke diesel engine are shown in Fig. 3. The nozzle is located near the center of a piston cavity, and has four injection holes. For the six-stroke diesel engine, one extra nozzle was added on the cylinder head as shown in Fig. 4. This extra nozzle was of the same design as that of the four-stroke engine.

Diesel fuel for the first combustion process was injected through this extra nozzle, and methanol for the second combustion process was injected through the center nozzle. Here, we denoted the injection timing of the four stroke diesel engine as Xi. The injection timings of the first and second combustion strokes for the six-stroke diesel engine are shown as Xi I and Xi II, respectively. Crank angle X was measured from the intake BDC. In the six-stroke engine, crank angle of the first combustion TDC is 180 degrees

. The conventional four-stroke diesel engine that was chosen as the basis for these experiments was a single cylinder, air cooled engine with 82 mm bore and 78 mm stroke. NO concentration was measured by a chemiluminescence’s NO meter, and soot emission was measured by a Bosch smoke meter.

Since combustion heats of diesel fuel and methanol are different, injection flow rates of the first and the second combustion processes are defined by the amount of combustion heat. Here, the supplied combustion heat for the first combustion process is denoted by QII. The second combustion stroke is denoted by QII. The ratio of QII to Qt (Qt = QI+QII ; supplied combustion heat per cycle) is defined as the heat allocation ratio αH:

αH  =             QII              =         QII

QI +QII                       Qt

SPECIFICATIONS OF THE TEST ENGINE:

Four stoke                                           Six stroke

Diesel Engine                                    Diesel Engine

Engine type		                       DI, Single cylinder, Air cooled, OHV

Bore x Stroke [mm]				           82 x 78

Displacement [cc]			                          412

Top Clearance [mm] 	                                                  0.9

Cavity Volume [cc]		                                       16

Compression ratio 		                                       21

Intake Valve Open		100 BTDC				70 BTDC

Intake valve Close		1400 BTDC				1450 BTDC

Exhaust Valve Open		1350 ATDC				1400 ATDC

Exhaust Valve Close		120 ATDC				30 ATDC

Valve Overlap			     220					  100

Rated power 			5.9 kW /3000rpm

PERFORMANCE OF SIX STROKE DIESEL ENGINE

COMPARISON WITH FOUR STROKE DIESEL ENGINE:

A four-stroke engine has one intake stroke for every two engine rotations. For the six-stroke engine, however, the intake stroke took place once for every three engine rotations. In order to keep the combustion heat per unit time constant, the combustion heat supplied to one six-stroke cycle should be 3/2 times larger than that of the four-stroke engine.

There are many ways to compare performance between the four-stroke and six-stroke engines. For this paper, the authors have chosen to compare thermal efficiency or S.F.C. at same output power. If the thermal efficiency was the same in both engines, the same output power would be produced by the fuels of equivalent heats of combustion. Therefore, in order to make valid comparison, fuels supplied per unit time were controlled at the same value for both engines and engine speeds were kept constant. In this section, fuel supplied for the engines was only a diesel fuel. Performance of the six-stroke engine was compared with that of the four-stroke engine under various injection timings.

Exhaust NO, soot emissions, and indicated torque of the four-stroke and the six-stroke engines are shown in Fig. 5. Detailed conditions for comparison of the four-stroke and six-stroke engines are listed in Table. 3. The heat allocation ratio of the six-stroke engine was set at αH = 0.5. Injection flow rate of fuel was Qt4 = 0.50 kJ/cycle for the four-stroke engine and Qt6 = 0.68 kJ/cycle for the six stroke engine. For six stroke engine, it meant that the amount of 0.34kj was supplied at each combustion process. At the viewpoint of combustion heat, 075 kj/cycle of heat should be supplied for the six stroke engine to make the equivalence heat condition

Injection timing of the four-stroke engine was changed from 160 degrees (200BTDC) to 180 degrees (TDC). For six -stroke engine, the injection timing of the first combustion process was fixed to 165 degrees (15°BTDC) or 174 degrees (6°BTDC), and the second injection timing was changed from 520 degrees (200BTDC) to 540 degrees (TDC).

DETAILED CONDITIONS OF COMPSRISION BETWEEN THE FOUR STROKE AND SIX STROKE DIESEL ENGINES AND PERFORMANCE OF ENGINE

|  | Four Stroke Diesel Engine | Six Stroke Diesel Engine |
| --- | --- | --- |
| Engine Speed Ne [rpm] | 2007 | 2016 |
| Supplied combustion heat per cycle Qt [kJ/cycle] | 0.50 | 0.68 |
| Supplied combustion heat per unit time Ht [kJ/s] | 8.36 | 7.62 |
| Intake air flow per cycle Ma [mg/cycle] | 358.7 | 371.4 |
| Injection quantity per cycle Mf [mg/cycle] | 11.8 | 16 |
| Excess air ratio λ | 2.40 | 1.83 |
| Intake air flow per unit time Ma’ [g/cycle] | 6.00 | 4.16 |
| Injection quantity per unit time Mf’ [g/sec] | 0.197 | 0.179 |
| Brake torque Tb [N-m] | 15.52 | 5.88 |
| Brake power Lb [kW] | 3.26 | 1.24 |
| B.S.F.C. b [ g / kW.h] | 217.9 | 520.3 |
| I.M.E.P. Pi [kgf / cm2] | 5.94 | 4.37 |

Figure 5 showed that indicated torque of the six-stroke engine was almost same level with that of the four-stroke engine under various injection timings. NO concentration in exhaust gas of the six-stroke engine was lower than that  of the four-stroke engine. NO emissions from both engines were reduced by the retard of injection timing. The effect of retard in the second injection timing of the six-stroke engine was similar to that of the retard in the four-stroke engine. For the six-stroke engine, from the comparison between Xi I = 165 degrees (15°BTDC) and Xi I = 174 degrees (6°BTDC), it seemed that the No reduction effect appeared with the timing retard in the first combustion process.							            Soot emission in the exhaust gas of the four-stroke engine was low level and it was not affected by the timing retard of injection. However, the level of soot emission from the six-stroke engine was strongly affected by the timing of the second injection. When the injection timing was advanced from 528 degrees (12°BTDC), it was confirmed that the soot emission was lower than that of the four-stroke engine. From numerical analysis, it was considered that the soot formed in the first combustion process was oxidized in the second combustion process. On the contrary, when the injection timing was retarded from 528 degrees (12°BTDC), soot emission increased with the timing retard. Then, it was considered that the increased part of the soot was formed in the second combustion process because an available period for combustion was shortened with the retard of injection timing.

Heat release rates of the four-stroke engines are shown in Fig. 6. Experimental conditions were Xi = Xi I = 170 degrees (100BTDC) and  Xi II=530 degrees (100BTDC). The heat allocation ratio of six stroke engine was  αH = 0.5.

The cylinder temperature and heat release rate shown in the figure were calculated from the cylinder pressure. The pattern of heat release rate in the first combustion stroke of the six-stroke engine was similar to that of the heat release rate of the four-stroke engine. It was the typical combustion pattern that contained a pre-mixed combustion and diffusion combustion. On the other hand, since an increase of cylinder temperature in the second combustion process was caused by the compression of the burned gas formed in the first combustion stroke, a pre-mixed combustion in the second combustion process was suppressed by a short ignition delay.

The maximum cylinder temperature in the first combustion process was lower than that in the four-stroke engine. It was caused by smaller amount of fuel which was injected in the first combustion process. Considering these results, it was proved that NO concentration in the exhaust gas was reduced by the decrease of the maximum cylinder temperature in the first combustion process and EGR effect in the second combustion process.

The performance of these two engines could be compared by Table. 3. Since B.S.F.C. of the six-stroke engine obtained by the brake power suffered, S.F.C. is compared with I.S.F.C. For the Xi = 163 degree (170BTDC), I.S.F.C. of the four-stroke engine was 177.2 g/kW.h.

On the other hand, for the Xi I = 165 d (15°BTDC) and Xi II = 523 degrees (170 BTDC), I.S the six-stroke engine was 163.3 g/kW.h. i.e. I.S.F.C. of the six-stroke engine was slightly lower than that of the four-stroke engine.

It was considered that this advantage in I.S.F.C. was caused by a small cut-off ratio of constant pressure combustion. Because, in the six-stroke engine proposed here, the fuel divided into two combustion processes resulted in a short combustion period of each combustion process. Furthermore, in the reduction of NO emission, the six-stroke engine was superior to the four-stroke engine as shown in Fig. 5.

EFFECT OF HEAT ALLOCATION RATIO :

Effect of heat allocation ratio on engine performance of the six-stroke engine is shown in Fig. 7. Injection conditions were Xi I = 170 degrees (100BTDC) and Xi II = 530 degrees (100BTDC). Total fuel at the combustion heat basis was Qt = 0.68 kJ/cycle. It meant a high load in this engine because the total excess air ratio was 1.83 as previously shown in Table 3.

The maximum value of the indicated torque appeared around αH = 0.5. NO concentration in exhaust gas was reduced by an increase of heat allocation ratio. In other words, NO emission decreased with an increase of the fuel of the second

combustion process.

In the case of αH = 0.5, there is a relatively long ignition delay in the first combustion process and pre-mixed combustion was the main combustion phenomena in it. NO of high concentration was formed in this pre-mixed combustion process. On the other hand, in the case of αH = 1, diffusion combustion was the main combustion phenomena and NO emission was low.

Soot emission in exhaust gas increased with an increase of heat allocation ratio. Since the injection flow rate in the second combustion process increased with an increase of the heat allocation ratio, the injection period increased with an increase of the heat allocation ratio. It caused the second combustion process to be long, and unburned fuel that was the origin of soot remained after the second combustion process.

Figure 8 shows the heat release rates on αH = 0.15 and αH = 0.85. For αH = 0.15, since injection flow rate in the first combustion process was high and injection period in it was long, the combustion period in the first combustion process became long as compared with case of αH = 0.85. On the other hand, for αH = 0.85, the combustion period in the second combustion process became long as compared with case of αH = 0.15. It was also observed that the long combustion periods in both the first and second combustion were caused by the long diffusion combustion. Further, diffusion combustion was the main combustion phenomena of the second combustion process.

When the heat allocation ratio was 0.85, the ratio of heat release rates between the first and second combustion should be 15 : 85, however the actual ratio obtained from the figure was 46 : 54. This inconsistency was caused from the drift of the base lines of the heat release diagrams. For αH = 0.15, the actual ratio of heat release rates was 73 : 27 with the similar reason.

The cylinder temperature for the αH = 0.15 condition was higher than that of the αH = 0.85 condition. This could be explained as follows. In the first combustion stroke, since the injection flow rate of αH = 0.15 was higher than that of αH = 0.85, the combustion temperature for the αH = 0.15 condition was higher than that of αH = 0.85. In the second compression stroke, since the high temperature burned gas was re-compressed, the temperature of αH = 0.15 was also higher than that of αH = 0.85. As a result, the temperature at the beginning of the second combustion stroke was high in αH = 0.15 condition as compared with αH = 0.85 condition. At the later stage of the second combustion, however, the opposite relationship between these two temperatures were observed, because the injection flow rate of the second combustion process was low in αH = 0.15 condition.

PERFORMANCE OF THE DUAL FUEL SIX STROKE DIESEL ENGINE

COMPARISION WITH DIESEL FUEL SIX STROKE DIESEL ENGINE:

Operating conditions of comparison between the diesel fuel and the dual fuel six-stroke engines are shown in Table. 4. Experimental conditions were Xi I= 170 degrees (100 BTDC), Xi II = 530 degrees (10o BTDC) and αH = 0.5.

In dual fuel six-stroke engine, diesel fuel and methanol were supplied into first and second combustion process, independently. Combustion heats supplied per one cycle of the diesel fuel and dual fuel six-stroke engines were same. The combustion heat supplied per one cycle was selected as Qt = 0.43 kJ/cycle under the middle load condition. Performance of the dual fuel six-stroke engine was compared with the diesel fuel six-stroke engine under various injection timings in the second combustion process. Results are shown in Fig. 9.

Indicated torques of both engines were revealed constant around 15 N.m. As a result, it could be concluded that states of combustion of the diesel fuel and the dual fuel six-stroke engines had similar contributions on the engine performance. NO emissions from the dual fuel six-stroke engine were lower than those of the diesel fuel six-stroke engine. This effect appeared prominently at the advanced injection timing of the second combustion. Further, NO concentrations of both engines were reduced by the injection timing retard in the second combustion.

Soot emission in the exhaust gas of the diesel fuel six stroke engine increased with a retard of the injection timing in the second combustion. For the dual fuel six-stroke engine, the exhaust level of soot was very low under various injection timings of the second combustion process. Soot was formed clearly by the combustion of diesel fuel in the first combustion process and it was oxidized in the second combustion process. Considering these results, it was possible to estimate that soot was almost oxidized by methanol combustion in the second combustion process. This estimation is supported by a dual fuel diesel engine operated with diesel fuel and methanol.

DETAILED CONDITIONS OF COMPSRISION BETWEEN THE DIESEL FUEL AND DUAL FUEL DIESEL ENGINES AND PERFORMANCE OF ENGINES UNDER αH = 0.5 AND MIDDLE LOAD

|  | Diesel Fuel Six Stroke Diesel Engine | Dual Fuel Six Stroke Diesel Engine |
| --- | --- | --- |
| Engine Speed Ne [rpm] | 2016 | 2003 |
| Supplied combustion heat per cycle Qt [kJ/cycle] | 0.43 |  |
| Injection quantity per cycle (First Combustion Stroke) Mf1 [mg/cycle] | 5.0 (Diesel Fuel) |  |
| Injection quantity per cycle (Second Combustion Stroke) Mf2 [mg/cycle] | 5.0 (Diesel Fuel) | 10.7 (Methanol) |
| Excess air ratio λ | 2.98 | 3.15 |
| Brake torque Tb [N-m] | 3.14 | 3.14 |
| Brake power Lb [kW] | 0.66 | 0.66 |
| B.S.F.C. b [ g / kW.h] | 610.9 | 952.9 |
| I.M.E.P. Pi [kgf / cm2] | 3.43 | 3.53 |
| Indicated torque Ti [N-m] | 14.70 | 15.12 |
| Indicated power Li [kW] | 3.1 | 3.17 |
| I.S.F.C. bi [g / kW.h ] | 130.1 | 198.4 |
| Indicated specific heat consumption bi’ [MJ /kW.h] | 5.59 | 5.43 |

Comparisons of both engines under a high load condition are shown in Fig. 10. The combustion heat supplied per one cycle was selected as Qt = 0.68 kJ/cycle under the high load condition. Indicated torques of both engines were also revealed constant around 20 N.m. NO concentration had the same tendency as the cases of the middle load. Soot emission level of the diesel fuel six-stroke engine was high in this high load condition. For the dual fuel six-stroke engine, however, soot was very low under various injection timings of the second combustion process.

The performance of these engines was compared in Table. 4. For the second combustion process, since combustion heats of diesel fuel and methanol were different, injection quantities of both engines were different. B.S.FC. and I.S.FC. of the dual fuel six-stroke engine was sensibly higher than that of the diesel fuel engine. To compare the performance of these engines, injection quantity of both engines was defined by an amount of combustion heat, and S.FC. Should be calculated from it. As a result, indicated specific heat consumption of the diesel fuel six-stroke engine was 5.59 MJ/kW.h, and that of the dual fuel six-stroke engine was 5.43 MJ/kW.h. For the high load conditions shown in Table. 5, the similar advantage of the dual fuel six-stroke engine was observed.

In order to confirm the advantage of dual fuel six-stroke engine, the performance of these engines was compared with four-stroke engine as shown in Table. 6. NO concentrations of the diesel fuel and the dual fuel six-stroke engines were improved with 85 - 90% as compared with that of the four-stroke engine. Soot emission of the diesel fuel six-stroke engine was much higher than that of the four-stroke engine. However, for the dual fuel six-stroke engine, soot level was very low. Furthermore, the indicated specific heat consumption of the diesel fuel and dual fuel six-stroke engine were lower than that of the four-stroke engine. Especially, for the dual fuel six-stroke engine, the indicated specific heat consumption was improved with 15% as compared with that of the four stroke engine. From these results, it could be confirmed that the dual fuel six-stroke engine was superior to the diesel fuel six-stroke engine, and also it was superior to the four-stroke engine.

|  | Diesel Fuel Six Stroke Diesel Engine | Dual Fuel Six Stroke Diesel Engine |
| --- | --- | --- |
| Engine Speed Ne [rpm] | 2016 | 2006 |
| Supplied combustion heat per cycle Qt [kJ/cycle] | 0.68 |  |
| Injection quantity per cycle (First Combustion Stroke) Mf1 [mg/cycle] | 8.0 (Diesel Fuel) |  |
| Injection quantity per cycle (Second Combustion Stroke) Mf2 [mg/cycle] | 8.0 (Diesel Fuel) | 17.2 (Methanol) |
| Excess air ratio λ | 1.86 | 1.93 |
| Brake torque Tb [N-m] | 6.08 | 6.18 |
| Brake power Lb [kW] | 1.28 | 1.30 |
| B.S.F.C. b [ g / kW.h] | 504.0 | 777.7 |
| I.M.E.P. Pi [kgf / cm2] | 4.56 | 4.75 |
| Indicated torque Ti [N-m] | 19.68 | 20.38 |
| Indicated power Li [kW] | 4.15 | 4.28 |
| I.S.F.C. bi [g / kW.h ] | 155.5 | 236.2 |
| Indicated specific heat consumption bi’ [MJ /kW.h] | 6.61 | 6.37 |

DETAILED CONDITIONS OF COMPSRISION BETWEEN THE DIESEL FUEL AND DUAL FUEL DIESEL ENGINES AND PERFORMANCE OF ENGINES UNDER αH =0.5 AND HIGH LOAD

|  | Four Stroke Diesel Engine | Diesel Fuel Six Stroke Diesel Engine | Dual Fuel Six Stroke Diesel Engine |
| --- | --- | --- | --- |
| NO [ppm] ( % improvement) | 768 | 113 (85.3%) | 90.5 (88.2%) |
| Soot [%] (%improvement) | 6.8 | 28.8 (- 323.5%) | 0 (100%) |
| Indicated specific heat consumption bi’ [MJ/kW.h] (% improvement) | 7.51 | 6.61 (12.0%) | 6.37 (15.2%) |

PERCENTAGE IMPROVEMENTS OF EXHAUST EMISSION AND INDICATED SPECIFIC HEAT CONSUMPTION

EFFECT OF INJECTION TIMING:

Performance of the dual fuel six-stroke engine under various injection timings in the second combustion process was investigated on middle and high load. Experimental conditions were Xi I = 170 degrees (100BTDC) and αH = 0.5.

Influence of injection timings of the second combustion on the engine performance is shown in Fig. 11. Performance of the dual fuel six-stroke engine under both load conditions had the similar tendency with the timing retard. NO concentrations in the high load condition were higher than those of the middle load condition. However, soot emission levels of both load conditions were extremely low under various injection timings of the second combustion.

EFFECT OF HEAT ALLOCATION RATIO:

Performance of the dual fuel six-stroke engine under various heat allocation ratios was investigated on middle and high load. Injection conditions were Xi I = 170 degrees (100BTDC) and Xi II = 530 degrees (100BTDC). Since the combustion heat of methanol was low, experimental range of heat allocation ratio was limited by the smooth operation of the engine. Only the range from αH = 0.25 to 0.75 (on Qt = 0.43 kJ/cycle), and from αH = 0 to 0.5 (on Qt = 0.68 kJ/cycle) could be tested.	.

The influence of heat allocation ratio on performance of the dual fuel six-stroke engine is shown in Fig. 12. Indicated torque increased with an increase of the heat allocation ratio. NO concentration in exhaust gas was reduced with an increase of the heat allocation ratio. Soot was very low, irrespective of the methanol flow rate. Even if the load condition was high, it was concluded that soot was practically eliminated by a small amount of methanol in the second combustion process (8% of total fuel).

ADVANTAGES
1. Reduction in fuel consumption by at least 40%
2. Two expansions(work/Power stroke) in six strokes
3. Dramatic reduction in pollution ( up to 65%)
4. Higher overall efficiency 
5. Lower engine temperature & noise level
6. Due to more air intake, the cooling system is improved
7. Better scavenging and more extraction of work per cycle
8. Less inertia due to lightness of moving parts 

DISADVANTAGES
1. Brake power & indicated power per cycle per cylinder is comparatively lesser
2. Engine size increases due to many number of cylinders & additional components
3. Higher manufacturing cost of six stroke engine 

APPLICATIONS
Automobiles, heavy goods, construction-site and farm vehicles. 
motor-pumps, generator sets, stationary engines, etc. intended for agriculture and industry, motor boats.

SIX STROKE DIESEL ENGINE

NOMENCLATURE

Ne  : Engine speed

X    : Crank angle

Xi   : Injection timing of the four-stroke diesel engine

αH    : Heat allocation ratio

Q    : Supplied combustion heat

Qt    : Supplied combustion heat per cycle

P    : Cylinder pressure

V    : Cylinder volume

Vs  : Stroke volume

Pi    : Indicated mean effective pressure (LM.E.P)

Ti    : Indicated torque

Li    : Indicated power

Tb   : Brake torque

Lb   : Brake power

Ht    : Supplied combustion heat per unit time

Ma  : Intake air flow per cycle

Ma' : Intake air flow per unit time

Mf  : Injection quantity per cycle

Mi   : Injection quantity per unit time

λ    : Excess air ratio

b    : Brake specific fuel consumption (B.S.F.C.)

bl    : Indicated specific fuel consumption (I.S.F.C.)

bi'  : Indicated specific heat consumption

CONCLUSION

The performance of the dual fuel six-stroke engine was investigated. In this dual fuel engine, diesel fuel was supplied into the first combustion process and methanol was supplied into the second combustion process where the burned gas in the first combustion process was re-compressed. The results are summarized as follows.

1. Indicated specific fuel consumption (I.S.F.C.) of the six-stroke engine proposed here is slightly lower than that of the four-stroke engine (about 9% improvement). NO and soot emissions from the six-stroke engine was improved as compared with four-stroke engine under advanced injection timings in the second combustion stroke.

2. For the dual fuel six-stroke engine, the timing retard and an increase of heat allocation ratio in the second combustion stroke resulted in a decrease of the maximum temperatures in the combustion processes. It caused the reduction of NO emission.

3. For the dual fuel six-stroke engine, soot was practically eliminated by a small amount of methanol in the second combustion process.

4. From the comparison of the performance between the dual fuel six-stroke and the four-stroke engine, it was concluded that indicated specific heat consumption of the dual fuel six-stroke engine was improved with 15% as compared with the four-stroke engine.

NO concentration of the dual fuel six-stroke engine was improved with 90%.

Furthermore, soot emission was very low in the dual fuel six-stroke engine.

- As the fuel in one cycle was divided into two combustion processes and the EGR

effect appeared in the second combustion process, the decreased maximum

cylinder temperature reduced NO concentration in the exhaust gas It was further

confirmed that soot formed in the first combustion process was oxidized in the

second combustion process .Therefore, a six stroke DI diesel engine has

significant possibilities to improve combustion process because of its more

controllable factors relative to a conventional four-stroke engine.

Considering these results, it was confirmed that the dual fuel six-stroke engine

was superior to the four-stroke engine.

REFERENCES

1. Tsunaki Hayasaki, Yuichirou Okamoto, Kenji Amagai and Masataka Arai

“A Six-stroke DI Diesel Engine under Dual Fuel Operation “ SAE Paper No

1999-01-1500

2. S.Goto and K.Kontani, "A Dual Fuel Injector for Diesel Engines", SAE paper, No. 851584, 1985

3. “Internal Combustion Engines “ A book by Mathur & Sharma.