ACKNOWLEDGEMENT

I dedicate this seminar work on the almighty god and to our beloved parents who helped me all the way throughout my life.

I also express my gratitude to late Sri.B.M.Shreenivasaiha and Shri B.S. Narayana who paved the way for B.M.S. College of engineering to become one of the prestigious and renowned institutions in the world.

In this regard I would also express our thankfulness and are indebted to Dr.K.Radhakrishna,Professor and Head ,Department of Mechanical Engineering,who has given me his kind and valuable guidance and encouraged me through the course of my seminar works.

I also express my thankfulness to Dr.M.Venkatesh, Principal,B.M.S. College Of Engineering, who gave me the back   support and encouragement during the course of the work.

Finally, I wish to express my gratitude to all my  respected teachers in our college and friends  who have given me their valuable guidance and had encouraged me throughout  the course of this work and all those who directly or indirectly helped me for the successful completion of my work.

JAGADEESH  D.L

(1BM04ME403)

## Contents

## Introduction							(3-4)

## History									(4-5)

## Theory of operation						(5-6)

## Jet engines								(6)

## Auxiliary power units						(7-8)

## Gas turbines for electrical power production		(8-9)

## Turbo shaft engines						(9)

## Scale jet engines							(9)

## Applications								(10-14)

    - Micro turbines

    - Gas turbines in vehicles

    - Naval use

    - Commercial Use

- Amateur gas turbines					(15)

- Advances in technology					(16)

- Advantages and Disadvantages				(17)

- Conclusion							(18)

- Reference								(19)

# Gas turbine

# Introduction

A gas turbine is an engine where fuel is continuously burnt with compressed air to produce a stream of hot, fast moving gas. This gas stream is used to power the compressor that supplies the air to the engine as well as providing excess energy that may be used to do other work.

The engine consists of three main parts. The compressor, the combustor and the turbine.

The compressor usually sits at the front of the engine. There are two main types of compressor, the centrifugal compressor and the axial compressor. The compressor will draw in air and compress it before it is fed into the combustion chamber. In both types the compressor rotates and it is driven by a shaft that passes through the middle of the engine and is attached to the turbine.

The combustor is where fuel is added to the compressed air and burnt to produce high velocity exhaust gas. Down the middle of the combustor runs the flame tube. The flame tube has a series of holes in it to allow in the compressed air. It is inside the flame tube that fuel is injected and burnt. There will be one or more igniters that project into the flame tube to start the mixture burning. Air and fuel are continually being added into the combustor once the engine is running. Combustion will continue without the use of the igniters once the engine has been started. The combustor and flame tube must be very carefully designed to allow combustion to take place efficiently and reliably. This is especially difficult given the large amount of fast moving air being supplied by the compressor. The holes in the flame tube must be carefully sized and positioned. Smaller holes around where the fuel is added provide the correct mixture to burn. This is called the primary zone. Holes further down the flame tube allow in extra air to complete the combustion. This is the secondary zone. A final set of hole just before the entry to the turbine allow the remainder of the air to mix with the hot gases to cool them before they hit the turbine. This final zone is known as the dilution zone. The exhaust gas is fed from the end of the flame tube into the turbine.

The turbine extracts energy from the exhaust gas. The turbine can, like the compressor, be centrifugal or axial. In each type the fast moving exhaust gas is used to spin the turbine. Since the turbine is attached to the same shaft as the compressor at the front of the engine the turbine and compressor will turn together. The turbine may extract just enough energy to turn the compressor. The rest of the exhaust gas is left to exit the rear of the engine to provide thrust as in a pure jet engine. Or extra turbine stages may be used to turn other shafts to power other machinery such as the rotors of a helicopter, the propellers of a ship or electrical generators in power stations.

Cold air is drawn in from the left into the compressor (blue). The compressed air (light blue) then goes into the combustor. From the outside of the combustor the air goes through holes (purple) into the flame tube (yellow). Fuel is injected (green) into the flame tube and ignited. The igniters are not show here. The hot exhaust gas flows from the end of the flame tube past the turbine (red) rotating it as it passes. From there the exhaust exits the engine. The turbine is connected via a shaft (black) to the compressor. Hence as the turbine rotates the compressor rotates with it drawing in more air to continue the cycle.

Simplified gas turbine diagram.

Obviously this is only a simplified view of a gas turbine engine and a working engine is much more complicated.

## History

1500: The "Chimney Jack" was drawn by Leonardo da Vinci which was turning a roasting spit. Hot air from a fire rose through a series of fans which connect and turn the roasting spit.
1629: Jets of steam rotated a turbine that then rotated driven machinery allowed a stamping mill to be developed by Giovanni Branca.
1678: Ferdinand Verbeist built a model carriage relying on a steam jet for power.
1791: A basic turbine engine was patented with all the same elements as today's modern gas turbines. The turbine was designed to power a horseless carriage.
1872: The first true gas turbine engine was designed by Dr F. Stolze, but the engine never ran under its own power.
1897: A steam turbine for propelling a ship was patented by Sir Charles Parsons. This principle of propulsion is still of some use.
1903: A Norwegian, Ægidius Elling, was able to build the first gas turbine that was able to produce more power than needed to run its own components, which was considered an achievement in a time when knowledge about aerodynamics was limited. Using rotary compressors and turbines it produced 11 hp (massive for those days). His work was later used by Sir Frank Whittle.
1914: The first application for a gas turbine engine was filed by Charles Curtis.
1918: One of the leading gas turbine manufacturers of today, General Electric, started their gas turbine division.
1920. The then current gas flow through passages was developed by Dr A. A. Griffith to a turbine theory with gas flow past airfoils.
1930. Sir Frank Whittle patented the design for a gas turbine for jet propulsion. His work on gas propulsion relied on the work from all those who had previously worked in the same field and he has himself stated that his invention would be hard to achieve without the works of Ægidius Elling. The first successful use of his engine was in April 1937.
1934. Raúl Pateras de Pescara patented the free-piston engine as a gas generator for gas turbines.
1936. Hans von Ohain and Max Hahn in Germany developed their own patented engine design at the same time that Sir Frank Whittle was developing his design in England.

Theory of operation

Gas turbines are described thermodynamically by the Brayton cycle, in which air is compressed isentropically, combustion occurs at constant pressure, and expansion over the turbine occurs isentropically back to the starting pressure.

In practice, friction, and turbulence cause:

a) non-isentropic compression - for a given overall pressure ratio, the compressor delivery temperature is higher than ideal.

b) non-isentropic expansion - although the turbine temperature drop necessary to drive the compressor is unaffected, the associated pressure ratio is greater, which decreases the expansion available to provide useful work. available to provide useful work.

c) pressure losses in the air intake, combustor and exhaust - reduces the expansion

As with all cyclic heat engines, higher combustion temperature means greater efficiency. The limiting factor is the ability of the steel, nickel, ceramic, or other materials that make up the engine to withstand heat and pressure. Considerable engineering goes into keeping the turbine parts cool. Most turbines also try to recover exhaust heat, which otherwise is wasted energy. Recuperators are heat exchangers that pass exhaust heat to the compressed air, prior to combustion. Combined cycle designs pass waste heat to steam turbine systems. And combined heat and power (co-generation) uses waste heat for hot water production.

Mechanically, gas turbines can be considerably less complex than internal combustion piston engines. Simple turbines might have one moving part: the shaft/compressor/turbine/alternator-rotor assembly (see image above), not counting the fuel system.

More sophisticated turbines (such as those found in modern jet engines) may have multiple shafts (spools), hundreds of turbine blades, movable stator blades, and a vast system of complex piping, combustors and heat exchangers.

As a general rule, the smaller the engine the higher the rotation rate of the shaft(s) needs to be to maintain tip speed. Turbine blade tip speed determines the maximum pressure that can be gained, independent of the size of the engine. Jet engines operate around 10,000 rpm and micro turbines around 100,000 rpm.

Thrust bearings and journal bearings are a critical part of design. Traditionally, they have been hydrodynamic oil bearings, or oil-cooled ball bearings. This is giving way to foil bearings, which have been successfully used in micro turbines and auxiliary power

Jet engines

Jet engines are gas turbines optimized to produce thrust from the exhaust gases.

## Auxiliary power units

Auxiliary power units (APUs) are small gas turbines designed for auxiliary power of larger machines, such as those inside an aircraft. They supply compressed air for aircraft ventilation (with an appropriate compressor design), start-up power for larger jet engines, and electrical and hydraulic power. These are not to be confused with the auxiliary propulsion units, also abbreviated APUs, aboard the gas-turbine-powered Oliver Hazard Perry-class guided-missile frigates. The Perrys' APUs are large electric motors that provide maneuvering help in close waters, or emergency backup if the gas turbines are not working.

Gas turbines for electrical power production

GE H series power generation gas turbine. This 480-megawatt unit has a rated thermal efficiency of 60% in combined cycle configurations.

Industrial gas turbines range in size from truck-mounted mobile plants to enormous, complex systems. They can be particularly efficient — up to 60% — when waste heat from the gas turbine is recovered by a heat recovery steam generator to power a conventional steam turbine in a combined cycle configuration. They can also be run in a cogeneration configuration: the exhaust is used for space or water heating, or drives an absorption chiller for cooling or refrigeration. A cogeneration configuration can be over 90% efficient. The power turbines in the largest industrial gas turbines operate at 3,000 or 3,600 rpm to match the AC power grid frequency and to avoid the need for a reduction gearbox. Such engines require a dedicated enclosure.

Simple cycle gas turbines in the power industry require smaller capital investment than coal, nuclear or even combined cycle natural gas plants and can be designed to generate small or large amounts of power. Also, the actual construction process can take as little as several weeks to a few months, compared to years for base load power plants. Their other main advantage is the ability to be turned on and off within minutes, supplying power during peak demand. Since they are less efficient than combined cycle plants, they are usually used as peaking power plants, which operate anywhere from several hours per day to a couple dozen hours per year, depending on the electricity demand and the generating capacity of the region. In areas with a shortage of base load and load following power plant capacity, a gas turbine power plant may regularly operate during most hours of the day and even into the evening. A typical large simple cycle gas turbine may produce 100 to 300 megawatts of power and have 35 to 40% thermal efficiency. The most efficient turbines have reached 46% efficiency.

## Turboshaft engines

Turboshaft engines are often used to drive compression trains (for example in gas pumping stations or natural gas liquefaction plants)and are used to power almost all modern helicopters. The first shaft bears the compressor and the high speed turbine (often referred to as “Gas Generator” or "N1"), while the second shaft bears the low speed turbine (or “Power Turbine” or "N2"). This arrangement is used to increase speed and power output flexibility.

## Scale jet engines

Also known as:

- Miniature Gas Turbines

- Micro-jets

Many model engineers relish the challenge of re-creating the grand engineering feats of today as tiny working models. Naturally, the idea of re-creating a powerful engine such as the jet, fascinated hobbyists since the very first full size engines were powered up by Hans von Ohain and Frank Whittle back in the 1930s.

Recreating machines such as engines to a different scale is not easy. Because of the square-cube law, the behaviour of many machines does not always scale up or down at the same rate as the machine's size (and often not even in a linear way), usually at best causing a dramatic loss of power or efficiency, and at worst causing them not to work at all. An automobile engine, for example, will not work if reproduced in the same shape at the size of a human hand.

With this in mind the pioneer of modern Micro-Jets, Kurt Schreckling, produced one of the world's first Micro-Turbines, the FD3/67. This engine can produce up to 22 newtons of thrust, and can be built by most mechanically minded people with basic engineering tools, such as a metal lathe.

## APPLICATIONS

## Microturbines

A micro turbine designed for DARPA by M-Dot

Also known as:

- Turbo alternators

- Gensets

- MicroTurbine® (registered trademark of Capstone Turbine Corporation)

- Turbogenerator® (registered tradename of Honeywell Power Systems, Inc.)

Microturbines are becoming wide spread for distributed power and combined heat and power applications. They range from handheld units producing less than a kilowatt to commercial sized systems that produce tens or hundreds of kilowatts.

Part of their success is due to advances in electronics, which allows unattended operation and interfacing with the commercial power grid. Electronic power switching technology eliminates the need for the generator to be synchronized with the power grid. This allows the generator to be integrated with the turbine shaft, and to double as the starter motor.

Microturbine systems have many advantages over reciprocating engine generators, such as higher power density (with respect to footprint and weight), extremely low emissions and few, or just one, moving part. Those designed with foil bearings and air-cooling operate without oil, coolants or other hazardous materials. Microturbines also have the advantage of having the majority of their waste heat contained in their relatively high temperature exhaust, whereas the waste heat of recriprocating engines is split between its exhaust and cooling system. [2] However, reciprocating engine generators are quicker to respond to changes in output power requirement and are usually slightly more efficient, although the efficiency of microturbines is increasing. Microturbines also lose more efficiency at low power levels than reciprocating engines.

They accept most commercial fuels, such as natural gas, propane, diesel and kerosene. They are also able to produce renewable energy when fueled with biogas from landfills and sewage treatment plants.

Microturbine designs usually consist of a single stage radial compressor, a single stage radial turbine and a recuperator. Recuperators are difficult to design and manufacture because they operate under high pressure and temperature differentials. Exhaust heat can be used for water heating, drying processes or absorption chillers, which create cold for air conditioning from heat energy instead of electric energy.

Typical microturbine efficiencies are 25 to 35%. When in a combined heat and power cogeneration system, efficiencies of greater than 80% are commonly achieved.

MIT started its millimeter size turbine engine project in the middle of the 1990's when Professor of Aeronautics and Astronautics Alan H. Epstein considered the possibility of creating a personal turbine which will be able to meet all the demands of a modern person's electrical needs, just like a large turbine can meet the electricity demands of a small city. According to Professor Epstein current commercial Li-ion rechargeable batteries deliver about 120-150 w-hr/kg. MIT's millimeter size turbine will deliver 500-700 whr/kg in the near term, rising to 1200-1500 whr/kg in the longer term.

## Gas turbines in vehicles

Rover JET1

Gas turbines are used on ships, locomotives, helicopters, and in tanks. A number of experiments have been conducted with gas turbine powered automobiles.

In 1950, designer F.R. Bell and Chief Engineer Maurice Wilks from British car manufacturers Rover unveiled the first car powered with a gas turbine engine. The two-seater JET1 had the engine positioned behind the seats, air intake grilles on either side of the car, and exhaust outlets on the top of the tail. During tests, the car reached top speeds of 140 km/h, at a turbine speed of 50,000 rpm. The car ran on petrol, paraffin or diesel oil, but fuel consumption problems proved insurmountable for a production car. It is currently on display at the London Science Museum. Rover and the BRM Formula One team joined forces to produce a gas turbine powered coupe, which entered the 1963 24 hours of Le Mans, driven by Graham Hill and Richie Ginther. It averaged 107.8 mph (173 km/h) and had a top speed of 142 mph (229 km/h). In 1971 Lotus principal Colin Chapman introduced the Lotus 56B F1 car, powered by a Pratt & Whitney gas turbine. Colin Chapman had a reputation of building radical championship-winning cars, but had to abandon the project because there were too many problems with turbo lag. The fictional Batmobile is often said to be powered by a gas turbine or a jet engine.

American car manufacturer Chrysler demonstrated several prototype gas turbine-powered cars from the early 1950s through the early 1980s. Chrysler built fifty Chrysler Turbine Cars in 1963 and conducted the only consumer trial of gas turbine-powered cars.

In 1993 General Motors introduced the first commercial gas turbine powered hybrid vehicle—as a limited production run of the EV-1. A Williams International 40 kW turbine drove an alternator which powered the battery-electric powertrain. The turbine design included a recuperator.

Gas turbines offer a high-powered engine in a very small and light package. However, they are not as responsive and efficient as small piston engines over the wide range of RPMs and powers needed in vehicle applications. Also, turbines have historically been more expensive to produce than piston engines, though this is partly because piston engines have been mass-produced in huge quantities for decades, while small turbines are rarities. It is also worth noting that a key advantage of jets and turboprops for aeroplane propulsion - their superior performance at high altitude compared to piston engines, particularly naturally-aspirated ones - is irrelevant in automobile applications. Their power-to-weight advantage is far less important. Their use in hybrids reduces the responsiveness problem, and the emergence of the continuously variable transmission may also help alleviate this problem. Another, recent, idea is the 'Multi-Pressure' turbine proposed by Robin Mackay of Agile Turbines. This concept is expected to provide three different power levels - each of them exhibiting high efficiency and low emission levels.

The MTT Turbine SUPERBIKE appeared in 2000 (hence the designation of Y2K Superbike by MTT) and is the first production motorcycle powered by a jet engine - specifically, a Rolls-Royce Allison model 250 turboshaft engine, producing about 283kW (380bhp). Speed-tested to 365km/h or 227mph (according to some stories, the testing team ran out of road during the test), it holds the Guinness World Records for most powerful production motorcycle and most expensive production motorcycle, with a price tag of US$185,000.

Use of gas turbines in military tanks has been more successful. In the 1950s, a Conqueror heavy tank was experimentally fitted with a Parsons 650-hp gas turbine, and they have been used as auxiliary power units in several other production models. The first production turbine tank was the Swedish Strv 103A. Today, the Soviet/Russian T-80 and U.S. M1 Abrams tanks use gas turbine engines. See tank for more details.

Several locomotive classes have been powered by gas turbines, the most recent incarnation being Bombardier's JetTrain. See Gas turbine-electric locomotive for more information.

### Naval use

Gas turbines are used in many naval vessels, where they are valued for their high power-to-weight ratio and their ships' resulting acceleration and ability to get underway quickly. The first gas-turbine-powered naval vessel was the Royal Navy's Motor Gun Boat MGB 2009 (formerly MGB 509) converted in 1947. The first large, gas-turbine powered ships, were the Royal Navy's Type 81 (Tribal class) frigates, the first of which (HMS Ashanti) was commissioned in 1961.

The Swedish Navy produced 6 Spica class torpedoboats between 1966 and 1967 powered by 3 Bristol Siddeley Proteus 1282, each delivering 4300 hp. They were later joined by 6 upgraded Norrköping class ships, still with the same engines. With their aft torpedo tubes replaced by antishipping missiles they served as missile boats until the last was retired in 1986.

The Finnish Navy issued two Turunmaa class corvettes, Turunmaa and Karjala, in 1968. They were equipped with one 16 000 shp Rolls-Royce Olympus TMB3 gas turbine and two Wärtsilä marine diesels for slower speeds. Before the waterjet-propulsion Helsinki class missile boats, they were the fastest vessels in the Finnish Navy; they regularly achieved 37 knot speeds, but they are known to have achieved 45 knots when the restriction mechanism of the turbine was geared off. The Turunmaas were paid off in 2002. Karjala is today a museum ship in Turku, and Turunmaa serves as a flotating machine shop and training ship for Satakunta Polytechnical College.

The next series of major naval vessels were the four Canadian Iroquois class helicopter carrying destroyers first commissioned in 1972. They used 2 FT-4 main propulsion engines, 2 FT-12 cruise engines and 3 Solar Saturn 750 KW generators.

The first U.S. gas-turbine powered ships were the U.S. Coast Guard's Hamilton-class High Endurance Cutters the first of which (USCGC Hamilton) commissioned in 1967. Since then, they have powered the U.S. Navy's Perry-class frigates, Spruance-class and Arleigh Burke-class destroyers, and Ticonderoga-class guided missile cruisers. USS Makin Island, a modified Wasp-class amphibious assault ship, is to be the Navy's first amphib powered by gas turbines.

### Commercial Use

Three Rolls-Royce gas turbines power the 118 WallyPower, a 118 foot super-yacht. These engines combine for a total of 16,800HP allowing this 118 foot boat to maintain speeds of 60 knots or 70mph.

Another example of commercial usage of gas turbines in a ship, are the HSS class fastcraft ferry fleet. HSS 1500-class Stena Explorer, Stena Voyager and Stena Discovery vessels use combined gas and gas (COGAG) setups of twin GE LM2500 plus GE LM1600 power for a total of 68,000kW. The slightly smaller HSS 900-class Stena Carisma, uses twin ABB–STAL GT35 turbines rated at 34,000kW gross.

## Amateur gas turbines

A popular hobby is to construct a gas turbine from an automotive turbocharger. A combustion chamber is fabricated and plumbed between the compressor and turbine. Like many technology based hobbies, they tend to give rise to manufacturing businesses over time. Several small companies manufacture small turbines and parts for the amateur. See external links for resources.

## Advances in technology

Gas turbine technology has steadily advanced since its inception and continues to evolve; research is active in producing ever smaller gas turbines. Computer design, specifically CFD and finite element analysis along with material advances, has allowed higher compression ratios and temperatures, more efficient combustion, better cooling of engine parts and reduced emissions. On the emissions side, the challenge in technology is actually getting a catalytic combustor running properly in order to achieve single digit NOx emissions to cope with the latest regulations. Additionally, compliant foil bearings were commercially introduced to gas turbines in the 1990s. They can withstand over a hundred thousand start/stop cycles and eliminated the need for an oil system.

On another front, microelectronics and power switching technology have enabled commercially viable micro turbines for distributed and vehicle power.

ADVANTAGES AND DISANDVANTAGES

ADVANTAGES

- Gas turbines when compared to other engines type have very good power to weight ratio.

- Gas turbine engines are compact in size.

- Excess heat generated by engines can be used for other applications. For example: Preheating of air before it enters the combustion chamber.

- Gas turbines have good thermal efficiency and therefore good fuel efficiency when in prolonged use under a constant load.

DISADVANTAGES

- Gas turbines are very expensive.

- The high engine speed which turbines run at causes problems when designing and manufacturing turbine parts .The harsh conditions within a gas turbine mean that the materials used and the design must be thought through.

- At low engine speed (idling) are when the load fluctuates the efficiency of gas turbines is very low.

- The very hot exhaust gases make gas turbines very unsuitable for many uses.

CONCLUSION

In order to understand where developments in industrial gas turbine evolution will takes place in next few years, one needs to see the evolution in context of market developments.

Currently, the power generation business sites at the bottom of the curve, yielding poor $/MW/h returns. This position is recovering and with most stringent and environmental legislation in 2007 and 2008. Eventually inhabitating the viability of alternative forms of generation, the gas turbine power generation operators needs to be ready to deal with these changes.

As the Market situation improves in 3-4 years time, operators will face new challenges. They will need to be able to satisfy the requirements driven by tightened environmental legislation. They will also need to deal with the new market opportunities arising from the void left by nuclear stations that will begin to lose their operating consent, and as aging coal power stations come to the end of their life span.

References

Reference Book Name                                   Author                 Publication

Elements of Gas turbine propulsion                   J.D.Mattingly                     1996,     									                   Mcgraw-Hill,Inc

WEBSITES

www.wikipedia.com/geothermal/overview.html

www.howstuffworks.com

http://encarta.msn.com/encyclopedia_761564074/gas_turbine.html

http://www.energyplanet.info/gas_turbines/